To assess normal respiratory function and the changes induced by pregnancy, we measured several key respiratory parameters, including inspiratory time (TI), expiratory time (TE), duty cycle (the ratio of inspiratory phase duration to the total duration of the respiratory cycle), inspiratory pressure (the pressure generated during inhalation that reflects the effort needed to draw air into the lungs), respiratory drive (the mean inspiratory pressure generated by the respiratory muscles during the inspiratory phase), and respiratory frequency (Figure 10). These parameters provide a comprehensive view of respiratory function, and their analysis helps to determine the extent of respiratory adaptations that occur during pregnancy.

To analyse these parameters, we developed a custom Python-based analysis pipeline in a Jupyter notebook (Perkinson, M. (2024). Respiratory Data Analysis Script (Version 1.1.1) [Software]. Unpublished), designed to process telemetry data collected from mice, including ITP, temperature, activity, and behavioural events. The first step involved retrieving and aligning the telemetry data with behavioural timestamps, ensuring that all physiological readings corresponded to specific time points of interest. The pressure data were then smoothed using a Savitzky-Golay filter to reduce high frequency, non-physiological, noise while preserving critical features such as respiratory peaks and troughs.

Rather than analysis of entire recordings we used time windows around specific behaviours (i.e., sleeping), incorporating buffer periods to capture transitions into and out of each behavioural state. Within each time window, downward peaks (representing drops in pressure during expiration) were detected using the find_peaks function from scipy.signal, which identifies local minima in the smoothed pressure signal. Pre-peaks, representing the time immediately before the drop in pressure, were identified by detecting the local maxima preceding each downward peak using the argrelextrema function from scipy.signal. These peaks and pre-peaks were used to calculate respiratory parameters.

We only included valid time windows containing at least 50 peaks, with interpeak intervals within two standard deviations of the median. If the interpeak interval exceeded this threshold, it was considered the start of a new period, and the validation process was repeated. These criteria ensured that only physiologically relevant data were retained, minimizing the impact of artifacts or missed respiratory peaks due to mouse movement. The processed data were organised into summary tables for subsequent statistical modelling, enabling a detailed analysis of respiratory function and its alterations during pregnancy.